import math

import re

from datetime import datetime, timedelta

import pandas as pd

import streamlit as st

# ============================================================

# PAGE CONFIG

# ============================================================

st.set_page_config(

page_title="Smart Room Digital Twin",

page_icon="🏠",

layout="wide",

)

# ============================================================

# CONSTANTS

# ============================================================

# 1 simulation cycle = 1 simulated minute

SIMULATION_STEP = 60.0

THERMAL_CAPACITY = 90_000.0       # J/K

UA_CLOSED = 40.0                  # W/K

WINDOW_UA_FACTOR = 2.5

HEATER_POWER = 2000.0             # W

LIGHT_POWER = 100.0               # W

LIGHT_K = 5.0                     # lux per dimmer %

DEFAULT_E_TARGET = 500.0          # lux

FROST_LIMIT = 7.0                 # °C

TEMP_DEADBAND = 0.5               # °C

OCCUPIED_SETPOINT = 21.0          # °C

VACANT_SETPOINT = 17.0             # °C

VACANCY_TIMEOUT = 300.0           # 5 minutes

HEATER_ANOMALY_TIME = 600.0       # 10 minutes

HEATER_MIN_RISE = 0.10            # °C

MODEL_TEMP_THRESHOLD = 0.50        # °C

MODEL_LIGHT_THRESHOLD = 50.0       # lux

TARIFF = 0.20                      # €/kWh

ALWAYS_ON_POWER = HEATER_POWER + LIGHT_POWER

# ============================================================

# PHYSICAL LAYER

# ============================================================

class PhysicalRoom:

"""Physical layer: room + sensors + actuators."""

def __init__(self):

# Sensors

self.T_in = 20.0

self.RH = 50.0

self.T_out = 12.0

self.E_day = 300.0

self.E = 300.0

self.occ = True

self.P_el = 0.0

# Actuators

self.heater_on = False

self.dimmer = 0.0

self.window_open = False

# Internal state

self.vacancy_seconds = 0.0

# Fault simulation

self.heater_fault = False

self.lighting_fault = False

def reset(self):

self.__init__()

def update_vacancy(self, seconds):

if self.occ:

self.vacancy_seconds = 0.0

else:

self.vacancy_seconds += seconds

def calculate_power(self):

heater_power = HEATER_POWER if self.heater_on else 0.0

if self.lighting_fault:

light_power = LIGHT_POWER

else:

light_power = LIGHT_POWER * (self.dimmer / 100.0)

self.P_el = heater_power + light_power

return self.P_el

# ============================================================

# VIRTUAL LAYER - THERMAL MODEL

# ============================================================

class ThermalModel:

def __init__(self):

self.C = THERMAL_CAPACITY

self.UA_closed = UA_CLOSED

def effective_UA(self, window_open):

return self.UA_closed * WINDOW_UA_FACTOR if window_open else self.UA_closed

def time_constant(self, window_open=False):

return self.C / self.effective_UA(window_open)

def steady_state_temperature(

self,

T_out,

heater_on,

window_open=False,

):

ua = self.effective_UA(window_open)

heater_power = HEATER_POWER if heater_on else 0.0

return T_out + heater_power / ua

def predict(

self,

T0,

T_out,

heater_on,

dt,

window_open=False,

):

ua = self.effective_UA(window_open)

tau = self.C / ua

heater_power = HEATER_POWER if heater_on else 0.0

T_ss = T_out + heater_power / ua

return T_ss - (T_ss - T0) * math.exp(-dt / tau)

def time_to_setpoint(

self,

T0,

T_out,

heater_on,

T_sp,

window_open=False,

):

ua = self.effective_UA(window_open)

tau = self.C / ua

heater_power = HEATER_POWER if heater_on else 0.0

T_ss = T_out + heater_power / ua

if heater_on and T_sp > T0 and T_sp < T_ss:

numerator = T_ss - T_sp

denominator = T_ss - T0

if denominator <= 0:

return None

ratio = numerator / denominator

if ratio <= 0 or ratio >= 1:

return None

return -tau * math.log(ratio)

return None

# ============================================================

# VIRTUAL LAYER - LIGHTING MODEL

# ============================================================

class LightingModel:

def __init__(self, k=LIGHT_K):

self.k = k

def predict(self, E_day, dimmer):

dimmer = max(0.0, min(100.0, dimmer))

return E_day + self.k * dimmer

def required_dimmer(self, E_day, E_target):

required = (E_target - E_day) / self.k

return max(0.0, min(100.0, required))

# ============================================================

# VIRTUAL LAYER - ENERGY MODEL

# ============================================================

class EnergyModel:

def __init__(self, tariff=TARIFF):

self.energy_kwh = 0.0

self.tariff = tariff

def update(self, power_w, dt_seconds):

self.energy_kwh += power_w * dt_seconds / 3_600_000.0

def cost(self):

return self.energy_kwh * self.tariff

def savings_kwh(self, elapsed_seconds):

baseline = ALWAYS_ON_POWER * elapsed_seconds / 3_600_000.0

return max(0.0, baseline - self.energy_kwh)

def savings_eur(self, elapsed_seconds):

return self.savings_kwh(elapsed_seconds) * self.tariff

# ============================================================

# REASONING LAYER

# ============================================================

class ReasoningEngine:

def __init__(self):

self.messages = []

self.anomalies = []

# One-cycle manual overrides

self.manual_heater_override = None

self.manual_dimmer_override = None

# Heater anomaly tracking

self.heater_on_seconds = 0.0

self.heater_on_start_temperature = None

def determine_setpoint(self, room, current_hour):

night = current_hour >= 23 or current_hour < 6

if not room.occ or night:

return VACANT_SETPOINT

return OCCUPIED_SETPOINT

def decide_heater(self, room, T_sp):

self.messages = []

T = room.T_in

T_out = room.T_out

# Rule 1: Frost protection

if T < FROST_LIMIT:

self.messages.append(

"Frost protection: T_in < 7°C → heater ON."

)

return True

# Rule 4: Free heat lockout

if T_out >= T_sp:

self.messages.append(

"Free-heat lockout: T_out >= T_sp → heater OFF."

)

return False

# Manual one-cycle override

if self.manual_heater_override is not None:

command = self.manual_heater_override

self.messages.append(

"Manual heater command → "

+ ("ON." if command else "OFF.")

)

return command

# Rule 2: Hysteresis

if T < T_sp - TEMP_DEADBAND:

self.messages.append(

f"Hysteresis: T_in < {T_sp - TEMP_DEADBAND:.1f}°C → heater ON."

)

return True

if T > T_sp + TEMP_DEADBAND:

self.messages.append(

f"Hysteresis: T_in > {T_sp + TEMP_DEADBAND:.1f}°C → heater OFF."

)

return False

self.messages.append(

"Hysteresis deadband: maintaining previous heater state."

)

return room.heater_on

def decide_lighting(self, room, E_target):

# Rule 5: Vacancy shutoff

if not room.occ and room.vacancy_seconds >= VACANCY_TIMEOUT:

self.messages.append(

"Vacancy timeout reached → dimmer 0%."

)

return 0.0

# Rule 7: Daylight sufficient lockout

if room.E_day >= E_target:

self.messages.append(

"Daylight sufficient → luminaire OFF."

)

return 0.0

# Manual one-cycle override

if self.manual_dimmer_override is not None:

dimmer = self.manual_dimmer_override

self.messages.append(

f"Manual dimmer command → {dimmer:.0f}%."

)

return max(0.0, min(100.0, dimmer))

# Rule 6: Daylight harvesting

dimmer = (E_target - room.E_day) / LIGHT_K

dimmer = max(0.0, min(100.0, dimmer))

self.messages.append(

f"Daylight harvesting → dimmer {dimmer:.0f}%."

)

return dimmer

def update_heater_anomaly(self, room, dt):

if room.heater_on:

if self.heater_on_seconds == 0.0:

self.heater_on_start_temperature = room.T_in

self.heater_on_seconds += dt

if (

self.heater_on_seconds >= HEATER_ANOMALY_TIME

and self.heater_on_start_temperature is not None

and room.T_in - self.heater_on_start_temperature < HEATER_MIN_RISE

):

message = (

"Heater anomaly: heater commanded ON for more than "

"10 minutes without sufficient temperature rise."

)

if message not in self.anomalies:

self.anomalies.append(message)

else:

self.heater_on_seconds = 0.0

self.heater_on_start_temperature = None

def detect_lighting_anomaly(self, room):

if (

room.lighting_fault

and not room.occ

and room.vacancy_seconds >= VACANCY_TIMEOUT

):

message = (

"Lighting anomaly: luminaire remains active "

"while the room is vacant."

)

if message not in self.anomalies:

self.anomalies.append(message)

def clear_manual_overrides(self):

self.manual_heater_override = None

self.manual_dimmer_override = None

# ============================================================

# DIGITAL TWIN

# ============================================================

class DigitalTwin:

def __init__(self):

self.physical = PhysicalRoom()

self.thermal_model = ThermalModel()

self.lighting_model = LightingModel()

self.energy_model = EnergyModel()

self.reasoning = ReasoningEngine()

# State variables

self.T_sp = OCCUPIED_SETPOINT

self.E_target = DEFAULT_E_TARGET

self.duty_cycle = 0.0

self.heater_runtime_seconds = 0.0

self.predicted_time_to_setpoint = None

# Residuals

self.temperature_residual = 0.0

self.illuminance_residual = 0.0

# Latest predictions

self.last_predicted_temperature = self.physical.T_in

self.last_predicted_illuminance = self.physical.E

self.health_status = "HEALTHY"

self.simulation_seconds = 0.0

self.history = []

# Simulated clock starts at 12:00

self.current_clock = datetime(2026, 1, 1, 12, 0, 0)

def reset(self):

self.__init__()

def clock_string(self):

return self.current_clock.strftime("%H:%M")

def advance_clock(self):

self.current_clock += timedelta(seconds=SIMULATION_STEP)

def process_command(self, text):

command = text.strip().lower()

if not command:

return "No command entered."

# Window

if re.search(r"\b(open|άνοιξε|ανοιξε)\b.*\b(window|παράθυρο|παραθυρο)\b", command):

self.physical.window_open = True

return "Window opened."

if re.search(r"\b(close|κλείσε|κλεισε)\b.*\b(window|παράθυρο|παραθυρο)\b", command):

self.physical.window_open = False

return "Window closed."

# Occupancy

if re.search(r"\b(vacant|empty|απουσία|απουσια|άδειο|αδειο)\b", command):

self.physical.occ = False

return "Room set to vacant."

if re.search(r"\b(occupied|presence|παρουσία|παρουσια|occupied)\b", command):

self.physical.occ = True

return "Room set to occupied."

# Heater

if re.search(r"\b(heater|θέρμανση|θερμανση)\b.*\b(on|ενεργοποίηση|άναψε|αναψε)\b", command):

self.reasoning.manual_heater_override = True

return "Manual heater ON command accepted for the next cycle."

if re.search(r"\b(heater|θέρμανση|θερμανση)\b.*\b(off|απενεργοποίηση|σβήσε|σβησε)\b", command):

self.reasoning.manual_heater_override = False

return "Manual heater OFF command accepted for the next cycle."

# Lights ON/OFF

if re.search(r"\b(lights?|φώτα|φωτα|luminaire|φωτισμός|φωτισμος)\b.*\b(off|κλείσε|κλεισε|σβήσε|σβησε)\b", command):

self.reasoning.manual_dimmer_override = 0.0

return "Manual lights OFF command accepted for the next cycle."

if re.search(r"\b(lights?|φώτα|φωτα|luminaire|φωτισμός|φωτισμος)\b.*\b(on|άναψε|αναψε|ενεργοποίηση)\b", command):

self.reasoning.manual_dimmer_override = 100.0

return "Manual lights ON command accepted for the next cycle."

# Dimmer percentage

dimmer_match = re.search(

r"(?:dimmer|brightness|φωτισμός|φωτισμος|ένταση|ενταση)\s*(?:to)?\s*(\d+(?:\.\d+)?)\s*%?",

command,

)

if dimmer_match:

dimmer = max(0.0, min(100.0, float(dimmer_match.group(1))))

self.reasoning.manual_dimmer_override = dimmer

return f"Manual dimmer command accepted: {dimmer:.0f}%."

# Outdoor temperature

temp_match = re.search(

r"(?:outside temperature|outdoor temperature|εξωτερική θερμοκρασία|εξωτερικη θερμοκρασια|outside|outdoor)\s*(-?\d+(?:\.\d+)?)",

command,

)

if temp_match:

self.physical.T_out = float(temp_match.group(1))

return f"Outdoor temperature set to {self.physical.T_out:.1f}°C."

# Queries

if re.search(r"\b(temperature|θερμοκρασία|θερμοκρασια)\b", command):

return f"Indoor temperature: {self.physical.T_in:.2f}°C."

if re.search(r"\b(humidity|υγρασία|υγρασια)\b", command):

return f"Relative humidity: {self.physical.RH:.1f}%."

if re.search(r"\b(energy|ενέργεια|ενεργεια|κατανάλωση|καταναλωση)\b", command):

return (

f"Energy: {self.energy_model.energy_kwh:.4f} kWh, "

f"cost: €{self.energy_model.cost():.4f}."

)

if re.search(r"\b(window|παράθυρο|παραθυρο)\b", command):

return "Window: OPEN." if self.physical.window_open else "Window: CLOSED."

if re.search(r"\b(occupancy|presence|παρουσία|παρουσια)\b", command):

return "Room is occupied." if self.physical.occ else "Room is vacant."

if re.search(r"\b(heater|θέρμανση|θερμανση)\b", command):

return "Heater: ON." if self.physical.heater_on else "Heater: OFF."

if re.search(r"\b(lights?|φώτα|φωτα|luminaire)\b", command):

return f"Luminaire: {self.physical.dimmer:.0f}%."

return (

"Command not recognized. Examples: "

"open window, close window, heater on, heater off, "

"lights on, lights off, dimmer 50%, vacant, occupied."

)

def run_cycle(self):

room = self.physical

dt = SIMULATION_STEP

# 1. Read sensors / update internal physical state

room.update_vacancy(dt)

# 2. Determine setpoint

self.T_sp = self.reasoning.determine_setpoint(

room,

self.current_clock.hour,

)

# 3. Reasoning / actuator commands

room.heater_on = self.reasoning.decide_heater(room, self.T_sp)

room.dimmer = self.reasoning.decide_lighting(room, self.E_target)

# 4. Virtual-layer one-step predictions

predicted_temperature = self.thermal_model.predict(

T0=room.T_in,

T_out=room.T_out,

heater_on=room.heater_on,

dt=dt,

window_open=room.window_open,

)

predicted_illuminance = self.lighting_model.predict(

room.E_day,

room.dimmer,

)

# 5. Physical evolution

# Heater fault: command is ON, but no thermal effect is produced.

actual_heater_on = room.heater_on and not room.heater_fault

ua = self.thermal_model.effective_UA(room.window_open)

heat_power = HEATER_POWER if actual_heater_on else 0.0

dT_dt = (

heat_power - ua * (room.T_in - room.T_out)

) / THERMAL_CAPACITY

room.T_in += dT_dt * dt

# Lighting fault: luminaire remains fully active.

if room.lighting_fault:

room.dimmer = 100.0

room.E = room.E_day + LIGHT_K * room.dimmer

room.E = max(0.0, room.E)

# 6. Power and energy

power = room.calculate_power()

self.energy_model.update(power, dt)

# 7. Anomaly detection

self.reasoning.update_heater_anomaly(room, dt)

self.reasoning.detect_lighting_anomaly(room)

# 8. Model residuals

self.temperature_residual = room.T_in - predicted_temperature

self.illuminance_residual = room.E - predicted_illuminance

if abs(self.temperature_residual) > MODEL_TEMP_THRESHOLD:

message = (

"Thermal model divergence: "

f"residual={self.temperature_residual:.2f}°C"

)

if message not in self.reasoning.anomalies:

self.reasoning.anomalies.append(message)

if abs(self.illuminance_residual) > MODEL_LIGHT_THRESHOLD:

message = (

"Lighting model divergence: "

f"residual={self.illuminance_residual:.1f} lux"

)

if message not in self.reasoning.anomalies:

self.reasoning.anomalies.append(message)

# 9. Time to setpoint

self.predicted_time_to_setpoint = self.thermal_model.time_to_setpoint(

T0=room.T_in,

T_out=room.T_out,

heater_on=room.heater_on,

T_sp=self.T_sp,

window_open=room.window_open,

)

# 10. Duty cycle

if room.heater_on:

self.heater_runtime_seconds += dt

total_time = self.simulation_seconds + dt

if total_time > 0:

self.duty_cycle = (

self.heater_runtime_seconds / total_time

) * 100.0

# 11. Health status

self.health_status = (

"ANOMALY" if self.reasoning.anomalies else "HEALTHY"

)

# 12. Save predictions

self.last_predicted_temperature = predicted_temperature

self.last_predicted_illuminance = predicted_illuminance

# 13. Advance simulation time

self.simulation_seconds += dt

self.advance_clock()

# 14. Save complete history

self.history.append(

{

"Time": self.clock_string(),

"Simulated min": self.simulation_seconds / 60.0,

"T_in": room.T_in,

"T_out": room.T_out,

"T_sp": self.T_sp,

"T_predicted": predicted_temperature,

"Temp residual": self.temperature_residual,

"RH": room.RH,

"E_day": room.E_day,

"E": room.E,

"E_target": self.E_target,

"E_predicted": predicted_illuminance,

"Light residual": self.illuminance_residual,

"Occupancy": room.occ,

"Heater": room.heater_on,

"Dimmer": room.dimmer,

"Window": room.window_open,

"Power W": room.P_el,

"Energy kWh": self.energy_model.energy_kwh,

"Health": self.health_status,

}

)

# One-cycle manual commands expire here.

self.reasoning.clear_manual_overrides()

def run_cycles(self, number):

for _ in range(number):

self.run_cycle()

# ============================================================

# SESSION STATE

# ============================================================

def session_state_is_valid(twin):

required_physical = [

"T_in", "RH", "T_out", "E_day", "E", "occ", "P_el",

"heater_on", "dimmer", "window_open", "vacancy_seconds",

"heater_fault", "lighting_fault",

]

required_twin = [

"thermal_model", "lighting_model", "energy_model",

"reasoning", "simulation_seconds", "history",

]

if twin is None or not hasattr(twin, "physical"):

return False

if not all(hasattr(twin.physical, attr) for attr in required_physical):

return False

if not all(hasattr(twin, attr) for attr in required_twin):

return False

return True

if "twin" not in st.session_state:

st.session_state.twin = DigitalTwin()

elif not session_state_is_valid(st.session_state.twin):

st.session_state.twin = DigitalTwin()

if "command_response" not in st.session_state:

st.session_state.command_response = ""

twin = st.session_state.twin

# ============================================================

# SIDEBAR

# ============================================================

with st.sidebar:

st.header("🌡️ Physical Layer")

twin.physical.T_out = st.slider(

"Outdoor temperature °C",

min_value=-10.0,

max_value=40.0,

value=float(twin.physical.T_out),

step=0.5,

)

twin.physical.RH = st.slider(

"Relative humidity %",

min_value=20.0,

max_value=80.0,

value=float(twin.physical.RH),

step=1.0,

)

twin.physical.E_day = st.slider(

"Daylight lux",

min_value=0.0,

max_value=2000.0,

value=float(twin.physical.E_day),

step=10.0,

)

twin.physical.occ = st.checkbox(

"Occupancy / Presence",

value=bool(twin.physical.occ),

)

st.divider()

st.header("🎯 Virtual Targets")

twin.E_target = st.slider(

"Target illuminance lux",

min_value=100.0,

max_value=1000.0,

value=float(twin.E_target),

step=10.0,

)

st.divider()

st.header("🧪 Fault Simulation")

twin.physical.heater_fault = st.checkbox(

"Simulate heater fault",

value=bool(twin.physical.heater_fault),

)

twin.physical.lighting_fault = st.checkbox(

"Simulate lighting fault",

value=bool(twin.physical.lighting_fault),

)

# ============================================================

# MAIN PAGE

# ============================================================

st.title("🏠 Smart Room Digital Twin")

st.caption(

"Digital Twin architecture: "

"Physical Layer → Virtual Layer → "

"Reasoning Layer → Actuators → Feedback"

)

# ============================================================

# DIGITAL TWIN CONTROLS

# ============================================================

st.subheader("🎛️ Digital Twin Controls")

st.info(

"1 cycle = 1 simulated minute. "

"The Digital Twin executes the closed loop automatically; "

"you do not need to enter a command for every cycle."

)

col1, col2, col3, col4 = st.columns(4)

with col1:

if st.button("▶️ Run 1 Cycle", use_container_width=True):

twin.run_cycles(1)

st.rerun()

with col2:

if st.button("⏱️ Run 1 Minute", use_container_width=True):

twin.run_cycles(1)

st.rerun()

with col3:

if st.button("⏱️ Run 10 Minutes", use_container_width=True):

twin.run_cycles(10)

st.rerun()

with col4:

if st.button("⏱️ Run 60 Minutes", use_container_width=True):

twin.run_cycles(60)

st.rerun()

col5, col6 = st.columns(2)

with col5:

if st.button("🔁 Run 60 Cycles", use_container_width=True):

twin.run_cycles(60)

st.rerun()

with col6:

if st.button("🔄 Reset", use_container_width=True):

st.session_state.twin = DigitalTwin()

st.session_state.command_response = ""

st.rerun()

st.divider()

# ============================================================

# NATURAL LANGUAGE COMMAND

# ============================================================

st.subheader("💬 Natural-Language Command")

command = st.text_input(

"Enter a command",

placeholder=(

"e.g. open window, heater on, lights off, "

"dimmer 50%, vacant"

),

)

if st.button("Send Command", use_container_width=False):

if command.strip():

response = twin.process_command(command)

st.session_state.command_response = response

# Run one closed-loop cycle after the external intervention.

twin.run_cycle()

st.rerun()

if st.session_state.command_response:

st.success(st.session_state.command_response)

# ============================================================

# CURRENT STATE

# ============================================================

st.subheader("📊 Current State")

m1, m2, m3, m4 = st.columns(4)

with m1:

st.metric(

"Indoor Temperature",

f"{twin.physical.T_in:.2f} °C",

)

with m2:

st.metric(

"Humidity",

f"{twin.physical.RH:.1f} %",

)

with m3:

st.metric(

"Illuminance",

f"{twin.physical.E:.0f} lux",

)

with m4:

st.metric(

"Electrical Power",

f"{twin.physical.P_el:.0f} W",

)

# ============================================================

# SYSTEM STATUS

# ============================================================

st.subheader("🟢 System Status")

s1, s2, s3, s4 = st.columns(4)

with s1:

st.metric("Health", twin.health_status)

with s2:

st.metric(

"Simulation Time",

f"{twin.simulation_seconds / 60:.0f} min",

)

with s3:

st.metric("Simulated Clock", twin.clock_string())

with s4:

st.metric(

"Target Temperature",

f"{twin.T_sp:.1f} °C",

)

# ============================================================

# ACTUATORS

# ============================================================

st.subheader("⚙️ Actuators")

a1, a2, a3 = st.columns(3)

with a1:

st.metric(

"Heater",

"ON" if twin.physical.heater_on else "OFF",

)

with a2:

st.metric(

"Luminaire",

f"{twin.physical.dimmer:.0f} %",

)

with a3:

st.metric(

"Window",

"OPEN" if twin.physical.window_open else "CLOSED",

)

# ============================================================

# REASONING LAYER

# ============================================================

st.subheader("🧠 Reasoning Layer")

if twin.reasoning.messages:

for message in twin.reasoning.messages:

st.write("•", message)

else:

st.write("No reasoning messages yet. Run a simulation cycle.")

# ============================================================

# VIRTUAL LAYER

# ============================================================

st.subheader("🧮 Virtual Layer")

v1, v2, v3 = st.columns(3)

with v1:

st.metric(

"Predicted Temperature",

f"{twin.last_predicted_temperature:.2f} °C",

)

with v2:

st.metric(

"Predicted Illuminance",

f"{twin.last_predicted_illuminance:.0f} lux",

)

with v3:

if twin.predicted_time_to_setpoint is None:

value = "N/A"

else:

value = f"{twin.predicted_time_to_setpoint / 60:.1f} min"

st.metric("Predicted Time to Setpoint", value)

# ============================================================

# MODEL PREDICTION AND RESIDUALS - CURRENT CYCLE

# ============================================================

st.subheader("📐 Model Prediction and Residuals")

r1, r2 = st.columns(2)

with r1:

st.metric(

"Temperature Residual",

f"{twin.temperature_residual:.3f} °C",

)

with r2:

st.metric(

"Illuminance Residual",

f"{twin.illuminance_residual:.1f} lux",

)

st.caption(

"Residual = actual simulated value − model predicted value."

)

# ============================================================

# ENERGY

# ============================================================

st.subheader("⚡ Energy and Cost")

e1, e2, e3, e4 = st.columns(4)

with e1:

st.metric(

"Energy",

f"{twin.energy_model.energy_kwh:.4f} kWh",

)

with e2:

st.metric(

"Cost",

f"€{twin.energy_model.cost():.4f}",

)

with e3:

st.metric(

"Theoretical Always-On Savings",

f"{twin.energy_model.savings_kwh(twin.simulation_seconds):.4f} kWh",

)

with e4:

st.metric(

"Heater Duty Cycle",

f"{twin.duty_cycle:.1f} %",

)

# ============================================================

# ANOMALIES

# ============================================================

st.subheader("🚨 Anomalies")

if twin.reasoning.anomalies:

for anomaly in twin.reasoning.anomalies:

st.error(anomaly)

else:

st.success("No anomalies detected.")

# ============================================================

# HISTORY

# ============================================================

st.subheader("📈 Simulation History")

if twin.history:

df = pd.DataFrame(twin.history)

st.dataframe(

df.tail(20),

use_container_width=True,

)

# --------------------------------------------------------

# 5.5.1 Temperature history

# --------------------------------------------------------

st.markdown("### Temperature History")

st.line_chart(

df.set_index("Simulated min")[

[

"T_in",

"T_out",

"T_sp",

]

]

)

# --------------------------------------------------------

# 5.4 Prediction vs Actual - Temperature

# --------------------------------------------------------

st.markdown("### Prediction vs Actual – Temperature")

st.line_chart(

df.set_index("Simulated min")[

[

"T_in",

"T_predicted",

]

]

)

st.markdown("### Temperature Residual")

st.line_chart(

df.set_index("Simulated min")[

[

"Temp residual",

]

]

)

# --------------------------------------------------------

# 5.5.2 Humidity

# --------------------------------------------------------

st.markdown("### Humidity History")

st.line_chart(

df.set_index("Simulated min")[

[

"RH",

]

]

)

# --------------------------------------------------------

# 5.5.3 Lighting history

# --------------------------------------------------------

st.markdown("### Illuminance History")

st.line_chart(

df.set_index("Simulated min")[

[

"E",

"E_day",

"E_target",

]

]

)

# --------------------------------------------------------

# 5.4 Prediction vs Actual - Illuminance

# --------------------------------------------------------

st.markdown("### Prediction vs Actual – Illuminance")

st.line_chart(

df.set_index("Simulated min")[

[

"E",

"E_predicted",

]

]

)

st.markdown("### Illuminance Residual")

st.line_chart(

df.set_index("Simulated min")[

[

"Light residual",

]

]

)

# --------------------------------------------------------

# 5.5.4 Energy

# --------------------------------------------------------

st.markdown("### Energy and Power History")

st.line_chart(

df.set_index("Simulated min")[

[

"Power W",

"Energy kWh",

]

]

)

else:

st.info(

"No simulation history yet. Run one or more simulation cycles."

)

# ============================================================

# FORMAL DIGITAL TWIN DESCRIPTION

# ============================================================

st.subheader("📘 Formal Digital Twin Representation")

st.markdown(

"""

The implemented Smart Room Digital Twin can be represented by the tuple:

**(P, A, X, R, M)**

- **P – Monitored parameters:** indoor temperature, relative humidity,

outdoor temperature, daylight, illuminance, occupancy and electrical power.

- **A – Actuators:** heater, dimmable luminaire and window.

- **X – State variables:** temperature setpoint, illuminance target,

actuator states, energy consumption, heater duty cycle,

predicted time to setpoint and health/anomaly status.

- **R – Rules:** frost protection, hysteresis, setback, free-heat lockout,

vacancy shutoff and daylight harvesting.

- **M – Models:** thermal model, lighting model and energy model.

"""

)

# ============================================================

# CLOSED-LOOP ARCHITECTURE

# ============================================================

st.subheader("🔄 Closed-Loop Architecture")

st.markdown(

"""

```text

┌──────────────────────────────┐

│       PHYSICAL LAYER         │

│                              │

│   Room + Sensors + Actuators │

└──────────────┬───────────────┘

│ measurements

▼

┌──────────────────────────────┐

│        VIRTUAL LAYER         │

│                              │

│   Thermal Model              │

│   Lighting Model             │

│   Energy Model               │

└──────────────┬───────────────┘

│ prediction

▼

┌──────────────────────────────┐

│       REASONING LAYER        │

│                              │

│   Rules + Anomaly Detection  │

└──────────────┬───────────────┘

│ actuator commands

▼

┌──────────────────────────────┐

│      PHYSICAL ACTUATORS      │

│                              │

│   Heater / Lights / Window   │

└──────────────┬───────────────┘

│ new physical state

▼

┌──────────┐

│ FEEDBACK │

└────┬─────┘

│

└──────────────► next cycle

"""

)

st.subheader("⏱️ Simulation Information")

st.write("**Cycle duration:** 60 seconds (1 simulated minute)")

st.write(

f"**Current simulated time:** "

f"{twin.simulation_seconds / 60:.0f} minutes"

)

st.write(

"**Digital Twin loop:** Sensors → Model → Reasoning → "

"Actuators → Physical evolution → Feedback"

)

st.write("**Thermal parameters:** C = 90 kJ/K, UA = 40 W/K")

st.write("**Heater power:** 2000 W")

st.write("**Luminaire power:** 100 W")

st.write("**Energy tariff:** €0.20/kWh")